It models year-by-year road roughness growth (IRI) and computes three user-cost components:

1. Vehicle operating cost (VOC)

2. Travel time cost

3. Accident cost

It supports:

1. One-year simulation

2. Multi-year simulation with discounted total cost (NPV-like)

Data flow in riindex.py is:

1. Start from current IRI and road/traffic condition inputs.

2. Compute dIRI.

3. Update IRI.

4. Convert IRI to speed.

5. Compute VOC, travel time cost, accident cost.

6. Sum annual cost.

7. In multi-year mode, discount annual cost and accumulate total.

It consists of:

- Core deterioration function:

- roughness_increment(): computes annual dIRI from structural, cracking, rutting, and environmental/age effects.

- Roughness update:

- update_iri(): returns new IRI as IRI{new}=IRI{old}+dIRI.

- Performance/cost functions:

- speed_from_roughness(): speed decreases exponentially with IRI and has a minimum value of 5 km/h.

- compute_voc(): simplified VOC equation using IRI and speed.

- travel_time_cost(): cost from travel time = distance/speed times value of time.

- accident_rate(): accident rate increases linearly with IRI

- accident_cost():, accident rate multiplied by exposure and unit cost.7

- Orchestration:

- simulate_one_year(): runs one annual step and returns outputs.

- run_simulation(): loops over years, recomputes all metrics each year, discounts annual cost, returns yearly rows + total discounted cost (=present value).

Key formulas used are:

- Roughness progression: dIRI=dRI_s+dRI_c+dRI_r+dRI_e

where components represent structural(s), cracking(c), rutting(r), and environmental/age effects(e).

dRI_s = Kgs * 134 * math.exp(Kgm * m * AGE3) * (1 + SNPK) ** (-5) * YE4

dRI_c = Kgc * 0.0066 * dACR

dRI_r = Kgr * 0.088 * dRDS

dRI_e = Kgm * m * RI_a

- Speed model: v=max (V_{free} * e^{-beta * IRI}, 5)

- Discounting in year t: DiscountFactor_t= 1 /{(1+r)^t}

DiscountedCost_t= AnnualCost_t * DiscountFactor_t